301 Munster Road

Fulham

SW6 6BJ

Tel Nr:
020-7381-1989

Fax Nr:
020-7385-4006
[image: image1.jpg]mrcarpet

DESIGNS FOR FLOORS

I.





	Mr A & Mrs M German-Ribon

	57 Napier Avenue

	Fulham

	London

	SW6 3PS


18th April 2013
ESTIMATE 
Dear Mr & Mrs German-Ribon,

Further to our recent site survey, please find herewith our quotation to supply and lay Karndean flooring at the above mentioned address as detailed below.
Kitchen, Dining Room & passage to hallway

To supply Karndean Van Gogh 3mm gauge ‘LVT’ Planks

1219mm x 178mm (48” x 7”) Colour French Oak VGW85T @ £31.50m²

Shutter off base of washing machine recess as necessary, 

Supply and lay Ardurapid water based screed to fill in deep voids in this recess,  

Supply Base 600 screed to approx 30mm depth to lower part of concrete sub-floor,

Supply and lay 4mm flooring grade plywood over entire area of builders plywood,

Secure plywood with 20mm ring shank nails @ 100mm centres,

Overhang existing builders plywood by approx 150mm in length,

Drill & plug overhanging area into concrete & secure with screws & adhesive, 

Supply and apply P131 neoprene primer to entire prepared area, 

Supply and lay stopgap 700 superflex screed (fibre glass based) to entire floor,

Fitting tiles to same by stick down installation, using approved HT adhesive,

Remove existing fridge & freezer from kitchen recess to hallway area as discussed, 

All other areas to remain clear of furniture at time of installation

Use existing stone thresholds at doorway connection with hallway

For the sum of: -

Inclusive of VAT @ 20.00%          £ 3,800.00

Please Note
As we have discussed during my previous site visits the existing base screed was installed by others and although we have been advised by your builder that a primer was laid before the base screed, we cannot accept responsibility for any failure of this area of sub-floor during our preparation work and installation of new tiles.

Also, as discussed previously the builder will return to site and remove the existing latex skim screed as this area has no primer laid beneath and is already blown! Also more fixings are required into the marine plywood laid by the builder as the floor is bouncing at present because it has not been fully secured to the joists beneath. These areas were pointed out and the floor areas marked, during our site visit dated 17th April 2013.

We trust the above meets with your approval and look forward to receiving your further instructions in due course.

Yours sincerely

Colin Symonds

Payment terms: 

50% Deposit with order 

Balance payment due on or before 1st day of installation 

Or 1 calendar month from date of order – which ever is sooner

Order subject to terms & conditions printed overleaf.

This document was produced electronically and is valid without a signature.

