
[image: image1.jpg]mrcarpet

DESIGNS FOR FLOORS

I.





70 Lower Richmond Road

Putney, London

SW15 1LL

Tel: 020 8789 3133

Fax: 020 8780 2140

email: putneybranch@mr-carpet.co.uk

ATTEND SHEET 

[image: image2.wmf]
Fitter:
Dave Gladwin 
Attend Date:
Wednesday 29th July 2009
Time:
1st job - 8.30am
Customer Name:
Mr & Mrs Robson-Hanafin (P23379)
Site Address:
82 Dora Road

Wimbledon
                                       SW19 7HH
Mobile No: 07780 605050 (Mrs) / 07771 704647 (Mr)

1) Replace Living Room filler as per original plans 
2) Stick gripper & finish fitting carpet at French doors

3) Re-stretch / relay carpet at French door end of room where lifted by Audio engineers

4) Replace top riser of basement stairs where it meets the wood flooring (ever so slightly short)

5) Take up 1st flight of stairs, move gripper closer together to make riser & tread carpets meet to get rid of shadow line in carpet. (If it works, do the same for all stairs)

6) Fit Ali S/Nap in between newel posts on 1st floor landing (10cm long)
7) Attend to L/H side of upper bullnose, remove fold & cut net up against door architrave. (Customer to fit small beading after if nec.)

8) Remove S/Nap @ bathroom doorway & fit Large Ali Zig Zag. If this won’t work, pull back the carpet to  show customer why not


