Avalon Carpet Tile & Flooring Pre-Installation guide for:

CARPET

Carpet is a fiber like your clothing; it must be cleaned and maintained to keep it looking good. All manufacturers require carpet to be cleaned

every 12 to 18 months. Depending on your selection you may need to clean your carpet more often. This is especially true of lighter colors. Most

manufacturers recommend hot water (steam) extraction for cleaning carpets. Please refer to your manufacturers warranty guide for specific

cleaning and maintenance requirements for your carpet.

Please be aware of the following that may exist before and after installation.

_____ DUST—Dust will be generated from ripping up and removing existing floor coverings, cutting and installing the material. Our installers will do

everything possible to minimize dust; however it is not possible to eliminate it. Avalon does not pay or reimburse for cleaning up dust.

_____ SEAMS—All carpets will show seams. There is no such thing as an invisible seam. Berbers, low profile, flat weave, pattern loops all will show

more than plush carpet. Lighting from windows will enhance the visibility of a seam. Your estimator will decide on the best layout and seam placement

for your carpet selection. Please review seam placement with your estimator before signing.

_____ SHEDDING—New carpet can and will shed. Fiber type, twist and construction will affect how much a carpet will shed. It is normal to see some

fibers in your vacuum for the first 6 months. Shedding is not a manufacturing defect.

_____ SHADING/TEXTURE—Carpet can appear to look and feel different once installed in your home. Dye-lots, lighting, paint color, furniture lay

out all will affect the appearance of your carpet. Please remember the carpet sample you are looking at is only a representative of what you will be getting,

not an exact match.

_____ ROLL MARKS/INDENTS—Sometimes carpets can have crush/roll marks that are noticed after the carpet is installed. This condition is not a

defect. Please be aware heavy objects, such as furniture can and will leave permanent dents in your carpet. As per the manufacturer, allow up to 60 days

for roll marks to relax.

_____ STAIR RUNNERS/STEPS—Pattern carpet/runners will not line up the same on each step unless extra carpet is ordered to specifically do so.

Not all stair cases are square and true. This may cause the pattern to drift side to side. Runners can be installed two ways: 1. “Waterfall” the carpet will

extend over the step nose and “fall” at an angle to the next tread. There will be a visible gap with this type of installation. If your steps have any decorative

trim installed under the nose of the step the “waterfall” method is the only recommended installation. 2. “Cap/band” the carpet will extend over the step

nose and be tacked under the nose. Please review your choices with your estimator. Once the carpet is installed on your step it can not be changed. Carpet

installed on steps is not covered by manufacturer's warranty.

_____ WOVEN/FLATWEAVE CARPETS—These carpets require special care. Some can not be vacuumed using a beater bar vacuum. Seams in these

carpets are very noticeable. Some products can not have cross seams or be seamed at all. Your estimator will review your selection and seam placement to

make sure it will fit your needs.

_____ PATTERN CARPETS—All pattern carpets have pattern matches. When figuring seams we need to add the appropriate match everywhere there

is a seam. This can add a substantial amount of carpet to your job. There is no such thing as a “square” room. Certain patterns can accentuate this, especially

hallways. Small print, diamonds, square patterns will show a wall running out of square.

_____ WALLS/BASEBOARDS/ACCESS—Carpet comes in rolls 12 to 15 foot long. If we are installing carpet in an area that has difficult access or

attempting to bring carpet into a basement or attic there will most likely be some marks and scuffs on your walls, baseboards and railings. We will do

everything possible to avoid marking your baseboards and walls. Avalon Carpet Tile & Flooring is not responsible for touching up, repainting or

reimbursing for light marks and scuffs due to access issues.

_____ PREPARATION—Please remove all personal items, clothing, toys, small furniture, breakables, remove china from hutch, empty bottoms of

closets, remove linens from beds, remove drawers from dressers, disconnect and remove any electronics, computers, TV’s. It is important to have this

completed before the installer arrives at your home. Avalon does not move delicate items such as grandfather clocks, pianos, large entertainment units and

pool tables. Please make arrangements in advance to have these items moved. The temperature must remain at a minimum of 65 degrees 48 hours before

and after installation.

_____ REMOVING DOORS—We will remove doors and re-hang them as needed however if they need to be cut the installer will leave them off so

you can arrange to have them cut, we do not cut doors.

_____ CLEAN UP/ODORS—Our installers will vacuum using a canister/shop vac vacuum. They will remove all scraps, tubes, plastic and wrappers.

They will leave behind any sizable pieces, door mat sized or larger at their discretion. Please be aware that your carpet may still have loose fibers, sprouts,

and fuzz. You will need to vacuum your new carpet a few times to completely remove these items. There may be dust generated from installing your new

carpet or from ripping up and removing your existing carpet. Neither Avalon nor the installers are responsible for cleaning up or removing dust. You may

notice your carpet or pad has a noticeable odor after installation. This is not a defect nor hazardous to you or your family. The odor should dissipate within

two weeks.

_____ ELECTRICAL/ALARM WIRES AND PIPES—Avalon is not responsible for cut, pierced or broken electrical/alarm wires or pipes that are

improperly placed/ran in walls, along baseboards, under floors and not in accordance with your local building code.

I understand I must be present when the installers arrive to review the job, confirm style, color, seam placement and pay my COD. Avalon is not

responsible for any claims due to color, style and seam placement once the carpet is installed. Please understand an extra service charge may apply

for any extra work incurred due to unforeseen problems with your sub-floor or lack of proper preparation leading to extra time spent on your job

by the installers.

Notes: ____________________________________________________________________________________________________________________

__________________________________________________________________________________________________________________________

Customer Signature: ________________________________________________________________ Date: __________________________________

PA Home Improvement #PA3758 NJ Home Improvement #NJ13VH01173600 Rev. 3.10

®
