Hygrometer Reading Report

(To Be Filled Out By Customer)

Customer Name: ______________________________________
Job Number: __________________________________________
Date Hygrometer Installed: _________/_________/_________
Basis of Test
The hygrometer measures the Relative Humidity (R.H.) of a small volume of confined air in equilibrium with the subfloor, but isolated from the surrounding air. The base can be considered dry enough to install most floorcoverings if a series of tests produce readings not exceeding 75% R.H.

Test Procedure
Once sufficient time has elapsed to allow the entrapped air to reach moisture equilibrium with the screed or base, the unit can be switched on.

For 50mm thick screeds, where the damp proof membrane is placed between the base and the screed, a period of at least 4 hours should be allowed before the first reading is taken.
Equilibrium can be assumed either when two readings taken at 4 hourly intervals show no change or if the instrument is left in position overnight.

For thicker constructions i.e. where the damp proof membrane is placed below the base slab, a period of 72 hours should be allowed to elapse before the first reading.
Equilibrium can be assumed when two consecutive readings taken at 24 hour intervals show no change.

	Date
	Time of Reading
	Reading Value

	12/7/09
	13:00
	75%

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	


Example:

