[image: image1.jpg]WESTEX

FINEST QUALITY CARPETS




	Westex Carpets Maintenance Instructions

A good programme of regular cleaning and maintenance extends the life and original appearance of your carpet.
Vacuum Cleaning

For loop pile qualities vacuum cleaners without a beater bar are recommended.  For cut pile qualities beater bars are recommended.  Choosing a cleaner with beater bar: for maximum effectiveness use a vacuum cleaner with adjustable, rotating brushes which will loosen deep rooted dirt.  The suction of all vacuum cleaners should be sufficient so that it can get through to the backing and pick up any dust and dirt settled there.  There should be a large closed filter and filter system so the dust doesn’t re-enter the room.  Whichever model of vacuum cleaner you decide upon, it should be regularly serviced.  Keep the suction hose and attachments free from foreign bodies and ensure that these do not become blocked as this could reduce the suction.  Examine the suction head for rough, sharp edges or bent metal which could damage the carpet.
High traffic areas such as corridors, stairs and entrance halls should be vacuumed daily.  The whole carpet should be vacuumed twice a week.  In this way dirt lying on the surface will be removed, avoiding it penetrating further into the pile.  Equally important is that dirt already lying deep in the pile is removed.  This is, however, not always so easy and very expensive.  Surface dirt can be removed while the vacuum cleaner is kept in the direction of the pile for a long time and then the opposite way.  Repeat as necessary.

Follow the manufacturer’s instructions, and change the filter when it is approx 60% full.  The machine’s suction can lessen, the fuller the bag is.


Deep cleaning

Daily vacuuming is the most important step for carpet cleaning, however deeper extraction cleaning takes care of removing stubborn and deep lying tough dirt.
Westex recommends cleaning products which are recognised by WoolSafe.  WoolSafe is an organisation active world-wide and test products for the professional cleaning and maintenance of carpets.  The cleaning substance receives the WoolSafe Certificate for appropriate suitability.  By passing all the WoolSafe test requirements manufacturers can display a certificate and the WoolSafe logo on their packaging.
Westex recommends a dry or wet extraction clean every 12-18 months as a minimum, and certainly before dirt becomes visible.
You should use a recognized professional carpet cleaning company to undertake the work. Wool requires special care.  Excessive rubbing and high temperatures on the wool fibres should absolutely be avoided.  Wool suffers if bleach or alkaline products are used, and should only be treated with neutral cleaning agents with a pH value between 5.0 and 8.  When using wet cleaning methods wool must be dried quickly, to prevent browning or yellowing. 


Do not wait until dirt is visible on the carpet.  If the carpet is already heavily and excessively dirty, the cleaning is all the more difficult and expensive, and as a result its life is considerably reduced.

Never use products intended for another purpose such as washing-up liquid, soap and household cleaners.  Even if the initial result of cleaning appears good, problems can occur rapidly such as quick re-soiling, bleeding of colour and shade of the carpet fibres as well as the backing. 


Tips for decreasing soiling

Doormats

It is recommended that doormats are laid at every entrance to the house, as these hold lots of the coarse dirt, and minimize the extent of soiling on your carpet.  A doormat should be large enough to remove a generous amount of dirt from the street from the soles of your shoes.   A doormat should be beaten out and cleaned at regular intervals.  Thus keeping the ability to take dirt in.
Protective measures

Westex carpets are impregnated with a protective substance, called Stain Resist, to prevent rapid soiling.  Although this treatment is already advantageous, it doesn’t offer 100% protection.  Due to the treatment becoming worn over time by mechanical wear and tear from walking on it.  Stains and soiling should be removed as soon as they occur.
Possible Appearances of the Carpet

Shading

Shading is also described as ‚Pile Reversal’, and wrongly misjudged and misinterpreted as watermarking.  Here certain areas in soft, velvet carpets appear lighter or darker than the surrounding areas.  This is however only an optical effect, which is caused by the different directions of the fibres, and becomes evident through light incidence.   In traffic areas or where the floor is uneven, such shading can appears more intense, however it is on no account concerning a fault in the carpet itself.

Changes to colour

The colour of a carpet can change for various reasons.  A more frequent reason is tracking, where the treading down of the fibres refracts the light differently and an optical difference in colour occurs.  Also light soiling and fading of dye due to UV-rays can lead to colour changes.  A conscientious undertaken professional clean will restore the appearance of your carpet, and revives the colour, provided that the dye has not decreased.
Soiling caused by draughts

At the edge of the carpet near walls, doorways and ventilation systems heavy soiling often appears.  They appear as grey streaks and contain dust particles carried in the air, which retained in the carpet fibres as a filter would.

Loss of fibres

When a carpet is newly-laid, some short fibres have a tendency to come off.  With wool these fibres build up considerably, although they weigh very little.  These appear for the most part in the first few weeks and mainly with cut pile carpets.  The fibres are visible as fluff on the surface of the carpet.  This is a normal phenomenon and shouldn’t be worried about, as it lessens.  Vacuum your carpet more gently in the first few weeks, and change the dust bag or empty the dust containment unit more frequently.
Stain treatment
Stains in carpets should always be treated immediately.  The earlier you fight the stain, the easier the removal. 
· Absorb liquids immediately with an absorbent cloth (kitchen paper, toilet paper, cotton towel, etc). (Please only absorb – never rub!).

· Solid substances (e.g. yogurt or similar) remove with a spoon or the back of a knife.
· Identify the stain, in order to select the correct treatment for the stain.

· Always work the stain from the outside inwards.

· Dab stains, NEVER rub.

· Take care using solvents, put on a cloth first and not directly onto the carpet.

· Ensure there is sufficient fresh air supply

· Always test the cleaner/stain remover in an inconspicuous area (even better on a carpet sample of offcut), never directly on the stain.

· Always thoroughly dab away the solvent.
· If possible, dry the treated area using a hairdryer.
The recommended stain removal suggestions contain the methods in the order in which the treatments should be attempted.
The table: ‚Easy stains’ contains those stains which are relatively easy to remove by the recommended methods.
The table: ‘Difficult stains’ contains those stains which are harder to remove and require repeated attempts using stronger stain removers.  With some stains it is probably impossible to fully remove them.  In this case, help from professional carpet cleaners is required.

Treatment methods
1. Dab stains away using white kitchen towel

2. Vacuum
3. Cold water
4. Warm water
5. Diluted Woolsafe approved carpet shampoo
6. Woolsafe approved stain remover for water soluble stains
7. Woolsafe approved stain remover for fatty/oily stains
8. Absorbent paper or paper tissue and hot iron
9. Chewing gum remover (solvent or frozen)
10. Nail varnish remover or acetone
11. Turpentine substitute
12. White spirit
13. Rub with a coin
14. Gently rub with coarse sandpaper
15. Call a Woolsafe professional carpet cleaner



    Easy stains
	
	1st Step
	2nd Step
	3rd Step

	Blood
	5
	6
	3

	Burn or scorch mark
	13
	14
	

	Butter
	7
	5
	

	Candle wax
	8
	7
	

	Chewing gum
	9
	
	

	Chocolate / cocoa
	5
	7
	6

	Cola
	3
	5
	6

	Cream
	7
	5
	

	Eggs
	5
	6
	

	Floor wax
	7
	5
	

	Friut juice
	3
	5
	

	Gravy / sauce
	5
	6
	

	Herbal tea
	3 
	6
	

	Metal polish
	5
	
	

	Mustard
	5
	6
	

	Oil / fat
	7
	5
	

	Emulsion paint
	3
	5
	

	Shoe polish
	7
	5
	

	Tea
	3
	5
	6

	Urine (fresh)
	3
	5
	15


    Difficult Stains
	
	1st Step
	2nd Step
	3rd Step

	Soft drinks
	3
	6
	

	Bleach
	3
	15
	

	Radiator fluid
	5
	6
	15

	Coffee
	6
	
	

	Furniture polish
	7
	5
	

	Felt-tip pen
	7 oder 11
	12
	5

	Glue / Adhesive
	10
	11
	

	Ink
	3
	6
	5

	Ball-point pen
	12
	
	

	Lipstick
	7
	
	

	Milk
	4
	5
	15

	Nail Varnish
	10
	
	

	Oil-based paint
	11
	
	

	Rust
	15
	
	

	Soot
	6
	5
	2

	Tar (Asphalt)
	7
	
	

	Urine (old)
	6
	15
	

	Vomit
	5
	6
	15

	Red wine
	1
	6
	

	
	
	
	


Please note, that the aforementioned cleaning methods are not guaranteed.

The methods are recommended, and should be tested on a sample or in an inconspicuous place.  
If you are in any doubt, please contact your local cleaning expert.

