Order no.
 R16615



Customer:


Hodson
Address:


20 Lauriston Rd




Wimbledon





SW19 





Contact on site: Charlie Hawkings
Tuesday 22nd May commence.

Top floor.
Stock Twist – Limestone to bed 5. Note position of seam. Section coming from landing cut out waste. Maybe tight across width so possible seam behind door.

Tiverton to bed 4 and dressing. Fit ramps to bed 4 and landing. Angle trims to be fitted onto tiles when sourced.

First floor.

Hardboard throughout including the half landing. Fit ramps as required.

Master bed and dressing. Natural Line – 02. See position of seam in dressing room doorway.

Stock Twist – Limestone to bed 2 and 3.

Ground floor.

Study. Lay hardboard but leave sufficient space (40cms) around fire hearth unfinished. This hearth requires additional work to improve the faces before carpet is fitted.
General.
The landings and stairs, bed 1 and study are the next stage as is the basement. To save time, gripper these areas in advance as the timing is very tight on the next stage fitting.
