Al
R16571 Remedial

There are several places where the carpet is not finished well.
The gripper under the radiator on the side wall needs repositioning or extra pinning.  The wall is wavy so the weave does not run exactly parallel and the carpet is not flat onto the gripper.
I notice on the same wall in the cupboard that the carpet is short by a row!
On the other side in the corner under the units. The carpet is clearly puckered and requires attention.

Please replace the tread of the bull nose which has a pressure indentation. Use the carpet lifted from the hall and stored in the living room.
I appreciate that the original instructions may not have been clear as to the whereabouts of all of the available carpet but this needs to be done all the same.

Any queries then call me.

Mike
