Fitting Instructions R16701 – Christopher Wren
Important please see plan for laying direction

Wood flooring being fitted to hall, dinning room & family room.

· Fit wood through doors in one piece – No expansion bars between hall, family room & hall, dining room. 
· Where wood meets tiles in kitchen fit twin bar between tiles and wood the wood is higher than the tiles. If the twin bar is on a slight slope it can snap. Can you put a bead of clear silicon under the tile side of the twin bar to stop it snapping at a later date.

· The join between the wood & tiles is 3.20m long. If you have to fit the bar in 2 pieces please put the long length nearest the door and the short end up by the window where the traffic across the bar is less.
· Please note: The bottom bull nose is NOT being carpeted. Please keep gap between wood & 1st riser as tight and even as possible.

Any problems ring DG for clarification on 07860 291565

