[image: image1.jpg]mrcarpet

DESIGNS FOR FLOORS

I.





32 Stadium Studios

Riverside Road

London

SW17 0BA

Tel: 020 8879 0485

Fax: 020 8947 5432

Email: davidguyan@mr-carpet.co.uk

R J Clyde (Builders) Ltd
Venlaw

Peters Lane

Holyport

Maidenhead

Berkshire

SL6 2HW

2nd September 2009

Dear Sirs


Estimate (subject to our standard terms & conditions)
Re: Gym off master bedroom, Halebourne House, Halebourne Lane, Chobham.

Further to our recent site survey I have pleasure in showing the cost to supply & lay Oak wood flooring to above area.

Materials selected

· W0017 Oak Rustic Matt Lacquered timber Plank size 189mm x 21mm x 1830mm @ £42.25 m2 + VAT.
· Heatflow .35 tog wood underlay 3mm thick.

· Oak expansion trims to all doorways and at entire width of patio doors.

· PVA adhesive as required.

Installation Method 
Wood to be installed as a floating floor on 3mm underlay, tongue & grooved glued with PVA adhesive. Wood expansion trims to be fitted at all transitions between materials in doorways and at patio doors to balcony.  

Wood to be installed BEFORE skirtings are fitted to allow for perimeter expansion. Skirtings to be fitted by others after installation.

Total Cost Labour & Materials for above installation
£3015.00 net + VAT 
Re: Storeroom off Gym, Halebourne House, Halebourne Lane, Chobham.
· Additional cost to install the above flooring to the store room off the Gym to the above specification if required.
Total Cost Labour & Materials for above installation
£810.00 net + VAT
If the store room is not ordered at the same time & fitted in a single visit with the gym then additional labour costs will be incurred. 
Continued Over
Important Notes for customer
1. Wood flooring is a natural product and colour, grain and knots may vary from the sample.

2. The under floor heating needs to be switched off for at least 48hrs before installation and not turned back on until 48hrs after installation is completed. When the heating is switched back on it should be set low and the temperature increased gradually over 2-3 days until it reaches ambient temperature.

3. When wood is fitted on under floor heating it can dry out when the heating is on. If this happens then small gaps may appear at the joints and this is perfectly normal and cannot be avoided with this type of installation.

4. The new screed needs to be fully dried before the installation of the wood. This can take up to 4 weeks per 25mm in normal circumstances but this may be reduced when the under floor heating is in use.

Payment Terms
· 50% deposit with order. Balance due 7 days after installation.

Delivery

· Delivery is approximately 10 – 14 days from receipt of order.
I trust this estimate meets with your approval and it is guaranteed for a period of 21 days from the above date. Thereafter it may be subject to change without notification if a manufacturer’s price increase prevails. If there are any further points you may wish to discuss or alternative options you may require a cost for please do not hesitate to contact me. 

Assuring you of our best attention at all times.

Yours Sincerely




David Guyan (contract sales)

_875940307.doc
�



�
















