[image: image1.jpg]mrcarpet

DESIGNS FOR FLOORS

I.





Health & Safety at Work

Warehouse Procedures 2007
Deliveries & Collections
When vehicles arrive at the warehouse to be loaded or unloaded please ensure that they park, whenever possible, allowing for pedestrians to pass without a restricted view of the loading & unloading procedures. Where possible use the appropriate Fork Lift for the safe loading & unloading of the vehicles. If goods have to be manually handled then the Lifting & Manual Handling procedures should be adhered to at all times and if in doubt do not lift any goods liable to injure yourself or anybody in the vicinity.

All goods delivered should be stored correctly in the appropriate racks or storage facility provided for each item.

Lifting & Manual Handling

Lifting, pushing or pulling any size of object the wrong way can cause serious back injuries, so the first thing you should try to do is see if there is a way of working where the load does not have to be moved at all. If it does have to be moved, avoid doing this manually if you can; see if you can use a piece of equipment, such as Palette Truck or Fork Lift, to do the work for you. 

 Moving a load by hand 

Remember, if you cannot move a load safely then you should not move it at all.

Where you cannot avoid moving a load by hand, a risk assessment must be carried out so that a way of doing the task safely can be worked out. 

Consider the following when making an assessment:

Only move something within your capacity. This varies from person to person due to size, fitness etc, but as a general rule if it does not feel comfortable then you should not move it by yourself.
                            Ask for help if the load is too awkward or heavy. 

Always think about the size and shape of the load and how you will have to carry it. Just because something has a handle does not necessarily mean that it is the best handhold for moving it. 

Think about where you are going to move the load to; will you need help to open doors?
Before you lift look for protruding sharp edges, notches, grease or anything, which may weaken your grip or injure your hands or legs. Do you need gloves to help you get a better grip? 

Carrying things at arms length is particularly bad for your back.

The kinetic lift 
However, even when you have considered all of this, if you do not know how to lift properly you are far more likely to injure your back. If you adopt the approach detailed below you will dramatically reduce your chances of being injured. This is called the kinetic lifting method and it helps you to make the most effective use of your muscles. 

Make sure that you have enough space to carry out a good lift.
Carrying out the kinetic lift 
Place your feet close to the load with one along side it and the other behind and settle into a comfortable and well-balanced position (Fig. 1). 

Lower your body and bend your knees in a smooth action, keeping your feet firmly on the floor (Figs. 2 & 3). The angle between your thigh and calf should not be less than 90 degrees because this gives your body greater mechanical advantage. 

Tilt the load forward to gain an idea of the weight (Fig. 4); if it is too heavy for you stop and ask for help. 

Get a good grip on the load with one hand on the bottom corner nearest to your body and the other hand on the opposite corner on the front of the load (Fig. 5). Make sure that your palms and fingers are in contact with as much of the load as possible. 

At the point of lifting, raise your head (Fig. 5). This action naturally puts your spine in the best position to carry out the lift and allows your legs to take the strain. It also means that you can see where you are going. 

You should then move upwards in a smooth continuous action (Figs. 6, 7 & 8). You must never lift and twist at the same time. If you want to change direction wait until you are upright and then move your feet first, your body will follow naturally. 

For smaller items hold the load close to your body at waist height and keep your elbows tucked in. 
[image: image6.png](fewrgt?



                             

If moving rolls of carpet then the above lifting actions must be followed but it is safer if the carpet is carried at shoulder height. If the carpet is being carried by two or more people always ensure that the carpet is not allowed to roll on your shoulder resulting in the other person making sudden moves to try to stop the carpet from falling. 

Remember a person in front of you carrying the roll cannot see your movements. Advise them verbally if you are struggling with the size, weight or bulk of a roll so in an emergency it can be dropped safely by both people and without injuring other staff or members of the public in the vicinity.
Moving a load by Fork Lift

Refer to separate section in this manual regarding the correct use of fork lift trucks.
Storage & Handling
Carpets & Vinyl Rolls: - Carpets & vinyl rolls should be stored in the racks provided. Wherever possible the Fork Lift truck should be used. If the fork lift truck is not suitable for loading or unloading carpets then you should assess your ability to lift the items BEFORE attempting to lift them. If in any doubt seek assistance before attempting to lift. Rolls that are carried by hand should not be put into racks higher than is comfortable for the people carrying the carpet. DO NOT stretch or climb on racks to load shelves above waist height. If any items are placed on shelves above waist height then the appropriate Step Ladders should be used.

Only the forklift truck fitted with the carpet boom should be used for this purpose.
Tiles, & Boxed items: - Tiles & Boxed Items should be stored in the racks or areas provided. Wherever possible the Fork Lift truck should be used. If the fork lift truck is not suitable for loading or unloading Tiles, & Boxed items then you should assess your ability to lift the items BEFORE attempting to lift them. If in any doubt seek assistance before attempting to lift. Boxes that are carried by hand should not be put into racks higher than is comfortable for the person carrying the boxes. DO NOT stretch or climb on racks to load shelves above waist height. If any items are placed on shelves above waist height then the appropriate Step Ladders should be used.

Only the forklift truck fitted with forks should be used for this purpose.

Adhesives/Screeds & Sealants: - Extra care should be taken when handling adhesives/Screeds & sealants as they may or may not be harmful if spilt.  If any items are spilt further information can be found on Mr Carpet Intranet in Shared Documents under Health and Safety or in the Health & Safety Manual held in Unit 32. The adhesives/Screeds & Sealants should only be stored on the racking provided in the ancillary areas in unit 32. If any items are placed on shelves above waist height then the appropriate Step Ladders should be used.

Under no circumstances should racks be climbed up for access to the higher shelves.
Hardboard, Gripper & Wood packs: - Hardboard & Gripper is unstable when the palettes they arrive in are opened. The gripper and Hardboard should be placed in the storage boxes provided. These boxes should only be moved using the Fork Lift truck. Any Hardboard, Gripper & Wood that are in opened palettes should be strapped with ratchet straps (at least 1 every 1m) to ensure they do not slip when being lifted. Under no circumstances should any palette be lifted off ground level that is not securely fastened either by manufacturers packing or Mr Carpets own ratchet straps.
 Only the forklift truck fitted with forks should be used for this purpose.

Door bars: - Door bars should be stored vertically in the racks specially constructed for this purpose. Where possible no more than 1 tube of each type of door bar should be opened at a time. This is because it is safer storing the bars in the tubes they are delivered in. Please ensure the tubes and loose bars are stored so they do not protrude further than the retaining posts between each section.
Machinery Operation
Cutting Machine Use: - The cutting machine should only be used by Mr Carpet Ltd’s staff that are trained for the correct use of the machine.  The machine is a mobile machine and should be moved into a safe position, before carpet is loaded on the machine, to the side of the main through route in unit 19. When the machine is in the correct position the castors should be locked before loading the machine. The appropriate roll of carpet should be loaded on to the machine by Fork Lift truck ensuring that it is lowered slowly on to the machine and without the Fork Lift truck contacting with the structure of the machine. The cut carpet should be removed from the machine (refer to the Storage & Handling section of this manual) and taken to the dispatch rack and the roll should be re-rolled and a minimum of 3 bands of tape should be applied to the rolls 4m wide and a minimum of 4 bands of tape should be applied to 5m wide rolls. The roll should then be placed into an appropriate storage rack (refer to the Storage & Handling section of this manual).
Only the forklift truck fitted with the carpet boom should be used for this purpose.

Use of Power Saw: - The electric chop saw in the door bar room is designed for cutting stock lengths of door bars down to individual door widths.  Protective glasses and ear defenders should be worn when using this machine. The fine fillings should be removed regularly using a vacuum cleaner or small brush and disposed of in the waste bins provided. No other items should be cut using this saw.
Maintenance of machinery: - All electrical machinery should be inspected prior to use and any damage or faults reported immediately before use. 

A thorough dust down and inspection should be carried out at least once a week. All cables, switches, electrical connections and safety guards etc. should be inspected and any sign of damage should be reported immediately to the warehouse manager. 

The warehouse manager should report all faulty or damaged equipment to a director of Mr Carpet Ltd and stop using the equipment until it has been passed as safe to do so by a director of Mr Carpet Ltd.
Under no circumstances should untrained personnel be allowed to use any of the above machinery.

Items in blue text have a separate sheet detailing the full procedures when using these items and should be read in conjunction with warehouse procedures 
[image: image2.jpg]mrcarpet

DESIGNS FOR FLOORS

I.





Health & Safety at Work

Safe Use of Forklift Trucks
Mr Carpet Ltd operates two forklift trucks. 

(1) Truck fitted with twin forks for lifting items on palettes only.

(2) Truck fitted with a carpet boom for lifting rolls of carpet and vinyl’s only.

Only the correct truck with the appropriate lifting apparatus should be used.

Before using any forklift truck the following points need to be checked by the user every time it is used before using.

(1) Are there any visible defects with the forklift truck?

(2) Do you hold the correct certificate of competence to use this truck?
(3) Are you authorised by a director of Mr Carpet Ltd to use this truck?

(4) Is the area you are operating the forklift truck in clear of pedestrians?

(5) Is the fork lift suitable for the load being carried and fitted with the correct lifting apparatus?
Any sign of damage of faults should be reported immediately to the warehouse manager. The warehouse manager should report all faulty or damaged equipment to a director of Mr Carpet Ltd and stop using the equipment until it has been passed as safe to do so by a director of Mr Carpet Ltd.
When using the fork truck
Provided you hold a current certificate of competence to drive the truck and are authorised by a director of Mr Carpet Ltd to use this truck then the following points must be observed.

Whilst Forklift in Motion

1. Customers ARE NOT allowed into forklift operating areas and should be asked to wait in their vehicle or in the office area if collecting or delivering goods. 

2. All pedestrian staff to stand clear of area where forklift is operating at all times. 

3. Always travel with the boom/forks as low to the ground as the load will allow when moving between warehouses, racks or lorries to increase vision and to  avoid overhead damage from an extended mast.

4. The driver should be aware in case anybody crosses the path or stands in front or behind whilst in motion.

5. Always look behind before reversing from a rack or lorry.

Whilst Forklift Unattended

1. When not in use forklifts to be left so as not to cause an obstruction to doors & walkways

2. Boom/Forks must be lowered to ground level.

3. Keys to be removed from ignition while forklifts are left unattended.

Regular Maintenance

It is the responsibility of all warehouse staff to check the following points with the forklift trucks.

Daily Checks

(1) The battery charge condition should be checked and if necessary the truck should be put on charge using the correct charger.

(2) The condition of the truck paying particular attention to the controls, tyres and hydraulic pipes should be checked and any defects reported immediately.

Weekly Checks

(1) The level of the batteries should be checked and if low should be topped up with distilled water only to the correct level. Care must be taken when filling the battery not to overfill or spill the water around the cells. 

(2) The condition of all the hydraulic pipes should be checked for wear or damage and any defects reported immediately.

(3) The condition of the tyres should be checked and while the tyres on the trucks used by Mr Carpet Ltd are solid any serious scrapes or damage should be reported immediately.

(4) The condition of the chains, wheels and mast should be checked for wear or damage and any defects reported immediately.

(5) The general condition of the truck including the effectiveness of the brakes, reversing beeper, the correct function of the controls and safety belt should be checked for wear or damage and any defects reported immediately.
[image: image3.jpg]mrcarpet

DESIGNS FOR FLOORS

I.





Health & Safety at Work

Manual Handling

Lifting, pushing or pulling any size of object the wrong way can cause serious back injuries, so the first thing you should try to do is see if there is a way of working where the load does not have to be moved at all. If it does have to be moved, avoid doing this manually if you can.

Where possible see if you can use a piece of equipment, such as a forklift truck, hand truck or trolley, to do the work for you.

Moving a load by hand 

Where you cannot avoid moving a load by hand, a risk assessment must be carried out so that a way of doing the task safely can be worked out. Although this responsibility rests with your manager, there will be many occasions when you have the necessary information to make an assessment yourself. 

Consider the following when making an assessment:

· Only move something within your capacity. 

· This varies from person to person due to size, fitness etc., but as a general rule if it does not feel comfortable then you should not move it by yourself. 

Ask for help if the load is too awkward or heavy. 
· Always think about the size and shape of the load and how you will have to carry it. Just because something has a handle does not necessarily mean that it is the best handhold for moving it.

· Think about where you are going to move the load to; will you need help to open doors etc.

· Before you lift look for protruding sharp edges, notches, grease or anything, which may weaken your grip or injure your hands or legs.

· Do you need gloves to help you get a better grip? 

Remember carrying things at arms length is particularly bad for your back.

The kinetic lift 

However, even when you have considered all of this, if you do not know how to lift properly you are far more likely to injure your back. If you adopt the approach detailed below you will dramatically reduce your chances of being injured. This is called the kinetic lifting method and it helps you to make the most effective use of your muscles. 

Carrying out the kinetic lift 

Place your feet close to the load with one along side it and the other behind and settle into a comfortable and well-balanced position (Fig. 1). 

Lower your body and bend your knees in a smooth action, keeping your feet firmly on the floor (Figs. 2 & 3). The angle between your thigh and calf should not be less than 90 degrees because this gives your body greater mechanical advantage. 

Tilt the load forward to gain an idea of the weight (Fig. 4); if it is too heavy for you stop and ask for help. 

Get a good grip on the load with one hand on the bottom corner nearest to your body and the other hand on the opposite corner on the front of the load (Fig. 5). Make sure that your palms and fingers are in contact with as much of the load as possible. 

At the point of lifting, raise your head (Fig. 5). This action naturally puts your spine in the best position to carry out the lift and allows your legs to take the strain. It also means that you can see where you are going. 

You should then move upwards in a smooth continuous action (Figs. 6, 7 & 8). You must never lift and twist at the same time. If you want to change direction wait until you are upright and then move your feet first, your body will follow naturally. 

Hold the load close to your body at waist height and keep your elbows tucked in. 

[image: image7.png](fewrgt?



                             

People who lift regularly must receive this training, although, because we all handle loads at work and at home everyday we would all benefit from the course. 

Make sure that you have enough space to carry out a good lift.

The Warehouse manager's role 

You are responsible for assessing the risk of your people being hurt from any manual handling tasks that they carry out. There are four main factors that you should consider when you carry out this assessment:

The person carrying out the lift, the task, the load and the environment. 

1. The person 

· The ability of a person to handle a load depends upon:                               Strength, height and degree of bodily movement. 

· General state of health; does the person have a history of back problems or have they just come back to work after illness?

· Training, experience and skill. 

· Other personal factors, such as pregnancy or disability. 

2. The task

· How often the load needs to be lifted? 

· How far it has to be carried? 

· Does it need to be held away from the body?

· Does it have to be lifted above shoulder height?

· Does it need strenuous pushing or pulling? 

3. The load

· How heavy is it?

· What is the packaging like; are there effective places for the hands?

· Is it very hot or very cold?

· Does it contain a corrosive substance?

· Are the contents liable to shift when being moved? 

4. The environment
· Is the floor uneven, slippery or unstable?

· Is there enough space?

· Is it hot cold or humid?

· What are the ventilation and lighting like?

· Is it windy? 

Things to consider in reducing manual handling accidents
Having carried out your assessment you should consider the following: 
· Mechanical aids – forklift truck, hand truck or trolleys etc.

· Training - do your people know how to lift?

· Reorganise the workplace - can you reduce the travel distance? For instance place photocopying paper next to the photocopier.

· The load - can you reduce the weight? Tell your supplier how you want to receive the goods.

· Storage - where possible store loads at waist height.

· Assistance - if one person cannot move a load safely assign another person to help.

· Coaching and supervision - reinforce good lifting, correct bad examples. Protective clothing - Are overalls required? People will understandably hold dirty loads away from their body. Are gloves required to give a better grip, or because the edges are sharp or the load is very hot or cold? 

Remember, in the final instance if your people cannot move a load safely then they should not move it at all.

[image: image4.jpg]mrcarpet

DESIGNS FOR FLOORS

I.





Health & Safety at Work

Use of Machinery – Non lifting equipment

Mr Carpet has two main items of machinery other than lifting equipment that are used on a regular basis.

1. Carpet Cutting Machine

2. Power Saw

Only authorised Mr Carpet personnel are to use any installed machinery.
Under no circumstances should untrained personnel be allowed to use any machinery.

Carpet Cutting Machine

The carpet cutting machine is to be used only for cutting rolls of carpet up to 5m wide. 

The following points should be observed when using.

· The cutting machine should only be used by Mr Carpet Ltd’s staff that are trained for the correct use of the machine.  

· The machine is a mobile machine and should be moved into a safe position, before carpet is loaded on the machine, to the side of the main through route in unit 19. 

· When the machine is in the correct position the castors should be locked before loading the machine. 

· The appropriate roll of carpet should be loaded on to the machine by Fork Lift truck ensuring that it is lowered slowly on to the machine and without the Fork Lift truck contacting with the structure of the machine. 
· The cut carpet should be removed from the machine and taken to the dispatch rack.
· The roll balance should be re-rolled and balances less than 10 m long should be check measured and the correct size entered on to the stock database.
· A minimum of 3 bands of tape should be applied to the rolls 4m wide and a minimum of 4 bands of tape should be applied to 5m wide rolls. 
· The roll should then be returned into an appropriate storage rack.
Only the forklift truck fitted with the carpet boom should be used for this purpose.

Power Saw

The power saw is to be used only for cutting stock lengths of door bars down to individual door widths. The following points should be observed when using.

· Protective glasses and ear defenders should be worn when using this machine.
· The fine fillings should be removed regularly using a vacuum cleaner or small brush and disposed of in the waste bins provided. 
No other items should be cut using this saw.

Maintenance of machinery

All electrical machinery should be inspected every time prior to use and any damage or faults reported immediately before use.

Regular Maintenance

It is the responsibility of all warehouse staff to check the following points with any machinery used in Mr Carpet’s Warehouse.

Daily Checks
· All cables, switches, electrical connections should be checked before use.

· Safety guards etc. should be inspected and any sign of damage should be reported immediately to the warehouse manager. 

Weekly Checks
· A thorough dust down and inspection should be carried out.

· All cables should be inspected and any cuts, splits etc should be reported immediately to the warehouse manager.

· Switches and electrical connections should be inspected and any damage should be reported immediately to the warehouse manager.

· Safety guards etc should be inspected and any damage should be reported immediately to the warehouse manager.

· The condition of any chains, belts or drive gears should be checked for wear or damage and any defects reported immediately to the warehouse manager.

The warehouse manager should report all faulty or damaged equipment to a director of Mr Carpet Ltd and stop using the equipment until it has been passed as safe to do so by a director of Mr Carpet Ltd.

The Warehouse manager's role 

You are responsible to ensure the above maintenance is carried out and any suspected or actual faults are reported immediately to a director of Mr Carpet Ltd.

Remember, in the final instance if you are not sure of the safety of any machine you should stop using it immediately and seek further advice.
[image: image5.jpg]mrcarpet

DESIGNS FOR FLOORS

I.





Health & Safety at Work

Safe Use of Steps & Working at Heights
Fixed steps are provided for access to high level racking in units 19 & 32. These steps should be the only ones used even in emergencies.  Do not borrow steps from adjacent companies that may be unsuitable for the purpose.
The safe use of step ladders starts with your attention to the conditions you are operating in before using them.  Also consider the following points before using the step ladder.
· Is the ladder sound and in good condition?

· Is there another member of staff in the building?

· Is the ladder suitable for the load being carried?
Any sign of damage should be reported immediately to the warehouse manager. The warehouse manager should report all faulty or damaged equipment to a director of Mr Carpet Ltd and stop using the equipment until it has been passed as safe to do so by a director of Mr Carpet Ltd.
Safe use of Step ladders. 
The following points should also be considered before use.

· Make sure the ladder is on firm level ground. 
· If you can, place the steps at right angles to the work, to avoid side thrusts tipping the steps over. If it is not possible, secure the steps before working, or get someone to steady them.
· Do not reach more than a comfortable arms length sideways and half this distance if pulling or pushing is necessary. Both feet should be firmly on the treads. 
· Watch out for doors, windows, pedestrians etc. Place a warning sign if necessary. 
· Always climb or descend the Steps facing them, holding on to the safety rail or rungs. 

�





�








S:\Health & Safety\Riverside\Riverside Road - Full Warehouse Manual.doc- 12 of 12

