	[image: image1.png]



	[image: image2.jpg]B | mrcarpet

. DESIGNS FOR FLOORS







308 Worple Road
Raynes Park

London
SW20 8QU
Tel Nr:
020-8947-7522
Email: raynesbranch@mr-carpet.co.uk

	Mr David Simpson

	

	info@skeyeview.co.uk

	

	


18th December 2020
Estimate – R18485
Re: 147 Ruden Way, Epsom Downs, KT17 3LW

Entrance hall, WC, kitchen, open plan living / dining room & rear extension

Uplift existing engineered wood flooring
Uplift existing engineered wood flooring / underlay, clear from site & dispose of same

Total Price for uplift & disposal

 
£577.50

Sub-floor preparation (Subject to further site survey once existing engineered wood flooring has been removed)

Supply & fit 6mm flooring grade plywood to the entire ground floor areas (excluding the new extension & the WC) 
& fix @ 4” centres using Spotnail fixings,

Supply & lay 6mm coat of Stopgap 1200 Pro latex screed over the concrete sub-floor in the new extension area to bring the floor level up to meet the plywood in the adjacent living room areas
Total Price for supply & installation

 
£1,587.10

Supply & fit LayRed tiles
Supply Moduleo LayRed Click Engineered vinyl tiles with built in acoustic underlay, Colour: Brio Oak 22927 @ £39.99m², 

Plank Size: 189mm x 1137mm x 6mm (with inbuilt underlay Decibel rating = ΔLw= 21dB),

Supply & fit Premier LVT trims (exact type / finish TBC) where the tiles meet the French doors at the rear & between the WC & hallway where there is currently a difference in height,
Supply labour to install the LayRed tiles directly over the newly prepared sub-floors leaving the required expansion gap around the perimeter (The required expansion gap will vary between 6mm - 15mm depending on the size of the area), 

All areas to be completely clear of furniture & appliances at the time of installation

Total Price for supply & installation


£4,286.34
Perimeter Beading
Supply & fit 15mm Primed White scotia beading at the bottom of the skirting boards to cover the perimeter expansion gap between the flooring & the skirting boards
Supply & apply Silicone sealant to fill smaller perimeter expansion gaps where beading is not suitable
Total Price for supply & installation

 
£469.88
Total price for supply & fit all of the above


£6,920.82
All prices include Vat @ 20%

Please see notes on following page:-
Please Note

Door trimming – Our fitters do not trim doors, therefore please ensure that all doors have sufficient clearance for your chosen flooring prior to installation
Existing sub-floor – As discussed during our conversation following my survey the existing sub-floor could not be fully assessed due to the existing Engineered Wood flooring still being in situ, therefore the prices quoted above are based on the assumption that the existing sub-floors are primarily timber floorboards (in reasonable condition) with the exception of the new extension, which we believe to be sound & level concrete, however should this prove NOT to be the case when the existing materials are removed, additional charges to cover additional or different types of floor preparation may apply?
Door architraves – Door architraves are often undercut when engineered wood flooring is installed. As the new flooring is not as thick as the current engineered wood please be aware that where architraves have been undercut there will be a gap between the bottom of the architraves & the new LVT flooring (The cost to repair or fill any such gaps is not included in our quotation)

Skirting boards – As the existing engineered wood flooring is thicker than the proposed LVT flooring some re-decorations may be required to the skirting boards & doorframes after the installation if the current paintwork does not go all the way to the floor (The cost of any such redecoration is not included in our quotation)

Perimeter beading – In order to cover the expansion gap between the LVT flooring & the skirting boards we have quoted to fix a 15mm Scotia beading at the bottom of the skirtings to cover the expansion gap. Please note that the beading will be primed White only & will therefore require further filling & painting to match it in with the skirtings boards (The cost of any such filling & painting is not included in our quotation)

Furniture – The above estimate is based on the installation being carried out when all areas are completely empty of furniture & kitchen appliances

Felt pads – We recommend applying felt pads (purchased separately) to all furniture legs to prevent the floor from scratching & we also recommend that suitable barrier mats be placed at all exterior access points to reduce scratching / soiling
January fitting offer – Please note that Vinyl flooring is NOT included in our January fitting promotion
Payment terms

50% Deposit payable with order & balance due 48 Hours before 1st day of installation (or 1 month after order placed if the fitting is delayed at customer’s request).

Order subject to terms & conditions printed overleaf

[image: image3.emf]T
This document was produced electronically and is valid without a signature.


