The Ridge, Ridgemount Road, Sunningdale

Further to my recent site survey I have pleasure in showing the cost to install wood flooring as detailed below.

Specification

Supply & apply 2 coats Sikabond Rapid liquid DPM to builders finish concrete sub floor.

To lay Supply Rustic Oak Carbonised engineered timber (Sample no W1025) board size 180mm x 20mm x 1900mm with UV oiled Flooring stuck direct with liquid batten flexible adhesive as required.

Finishes (as discussed with Rob Stone on site)
Fire hearths - 80 -90mm wide reduced width board mitred around fire hearth

Patio windows – Oak stop end expansion bar to be stained to match and fitted as required at door thresholds.

Electricity floor boxes – Wood to be cut into raised external frame set at 23mm higher than existing concrete floor and wood installed into top of internal frame. (Frame to have a 21mm minimum inset.

Tiled floor finishes – Tiled floors at adjacent rooms to wooden areas to be finished with metal up stand to edge of tiles. Wood flooring to be installed with a “credit card gap” against metal up stands. Minimum height of finished tiles to be 23mm above finished screed level. No allowance for transitional trims has been made in the following costings if there are differences in height between the wood and tiled areas and are chargeable as extra if required.

Expansion gaps at edges – Floors to be installed prior fitting of skirting. Expansion gaps to be covered by skirting installed by others.

Plot No:
Areas Included

1
Guest Bedroom & Drawing Room

2
Dining Room & Drawing Room

2
Dining Room & Drawing Room

4
Guest Bedroom & Drawing Room

5
Dining Room & Drawing Room

6
Dining Room & Drawing Room

7
Dining Room & Drawing Room

8
Dining Room & Drawing Room

9
Dining Room & Drawing Room Including Snug

10
Dining Room & Drawing Room Including Snug

