Living Room, 2 Bedrooms & Hallway (Waste carpet to cupboards)
To supply Stainfree Twist, 100% Polypropylene carpeting, 5 metre wide, Colour: STA-007,

Standard PU Foam underlay & Paperfelt Interliner to be laid beneath,

Fitting to same by gripperods installation,

Supply & fit standard aluminium doorbars at doorways,

Seam carpets to hallway recess outside bedroom 1 & inside cupboard to bedroom 1,

Supply & fit 1 x 9mm Easyshim ramp to raise height of carpet at junction with tiles in WC, 

Areas to be clear of furniture at time of installation

Kitchen & Bathroom

To supply Moduleo ‘Select’, 2.35mm gauge Luxury Vinyl tile Flooring, 

Plank size: 196mm x 1320mm Colour: Classic Oak 24932,

Supply and apply P131 primer over existing concrete sub-floor,

Supply and lay 5mm coat of Stopgap 1200 Pro latex screed to prepare sub-floor,

(Builder to remove existing ceramic tiles & fill in any large holes to provide a reasonably flat sub-floor for us to apply the final coat of latex screed)

Fitting to same by stick down installation using approved adhesive,

Appliances to remain out of the are until after the flooring is laid

Please Note

The fitters do not trim doors, therefore please ensure all doors are trimmed to the correct height prior to the installation.
Fitting Dates
Phase 1 – Thursday 5th May 2016 (Screed)
Phase 2 – Friday 6th May 2016 (Fit LVT)
Phase 3 – Wednesday 11th May 2016 (Fit Carpets)
£1,734.00 Deposit paid by Visa Debit on 19/04/16 with thanks,
£1,734.00 Balance payment on or before 1st day of installation or 1 month after order placed if the installation is delayed at the customer’s request.
