Stoddard VELLUTO

Product Specification Sheet

| Pile Content : | 100% 2-ply solution dyed nylon (Stainmaster ®) |
| --- | --- |
| Construction : | Velvet |
| Machine Gauge : | 1/10” |
| Widths Available: | 4m & 5m |
| Number of Colours: | 15 |
| Backing: | Secondary |
| Colour Names: | Antique Slate, Battleship, Biscotti, Café au Lait, Charcoal, Chartreuse, Décor, Duck Egg, Gunboat, Hint of Sage, Irish Peat, Lace, Machiatto, Shadow, Tyrolean Teal |

|  | Lusso | Contrato |
| --- | --- | --- |
| Pile Height: | 16mm | 11mm |
| Total Height: | 18mm | 13mm |
| Pile Weight | 65oz / 2200gm | 34oz / 1150 gm |
| Total Weight | 3170 gm | 2015 gm |
| Tog Rating | 1.6 | 1.2 |
| Suitability: | Heavy Domestic | Heavy Domestic |

INSTALLATION

To British Standard BS5325:1996 – Code of Practice for the Installation of Textile Floorcoverings.

MAINTENANCE

For day to day cleaning, use of a vacuum cleaner is recommended. Professional Cleaning by Dry Cleaning Extraction or Hot Water Extraction is recommended to prolong the life of the carpet.

STAINMASTER carpets resist most dirt, food and drink stains, while the specially engineered nylon fibres give an incredibly soft and luxurious feel. The fibre also retains its original appearance, resisting crushing and matting, which helps STAINMASTER carpets to keep looking good for a long time.

All cut pile textures can show colour variation where the lay direction of the pile varies in adjacent areas. This is not a manufacturing fault.